## Practical lesson №8. Gaussian and Laplace pyramids of image

Task №1

Build a Gaussian pyramid of the image with at least five layers. Visualize the resulting images and the frequency amplitudes on each level of the pyramid. Make sure that the frequency range is narrowed on each layer. Build a pyramid for three different values of the Sigma of the Gaussian nucleus. For the convenience of experimentation, define a separate function (build_gauss_pyramid) for building a Gaussian pyramid with the parameters img (the image on which the pyramid is built), sigma (the parameter of the Gaussian kernel), n_layers (the number of layers of the pyramid), which returns a list of required images.

from skimage.io import imread, imshow
from skimage.filters import gaussian
from numpy.fft import fft2, fftshift, ifft2
import numpy as np
import matplotlib.pyplot as plt
import cv2
from skimage import img_as_float

def build_gauss_pyramid(img, sigma, n_layers):
    FOURIER_IMG = fft2(img)
    filt_gauss = np.zeros(img.shape)
    filt_gauss[img.shape[0] // 2, img.shape[1] // 2] = 1
    IMG_GAUSS = gaussian(filt_gauss, sigma=sigma)
    FOURIER_GAUSS = fft2(IMG_GAUSS)
    imgs = []
    fouries = []
    for i in range(n_layers):
        FOURIER_BLUR_IMG = abs(FOURIER_GAUSS)* FOURIER_IMG
        FOURIER_IMG=FOURIER_BLUR_IMG
        imgs.append(ifft2(FOURIER_BLUR_IMG).real)
        fouries.append(FOURIER_BLUR_IMG)
    return imgs, fouries

if __name__ =="__main__":
    src = img_as_float(imread('lena1.jpg', as_gray=True))
    plt.figure(figsize=(25, 17))
    imgs, four  = build_gauss_pyramid(src, 2, 5)
    plt.subplot(341)
    imshow(src)
    plt.subplot(342)
    FOURIER_IMG = fft2(src)
    imshow(np.log(1 + abs(fftshift(FOURIER_IMG))))
    plt.subplot(343)
    imshow(imgs[0])
    plt.subplot(344)
    imshow(np.log(1 + abs(fftshift(four[0]))))
    plt.subplot(345)
    imshow(imgs[1])
    plt.subplot(346)
    imshow(np.log(1 + abs(fftshift(four[1]))))
    plt.subplot(347)
    imshow(imgs[2])
    plt.subplot(348)
    imshow(np.log(1 + abs(fftshift(four[2]))))
    plt.subplot(349)
    imshow(imgs[3])
    plt.subplot(3,4,10)
    imshow(np.log(1 + abs(fftshift(four[3]))))
    plt.subplot(3,4,11)
    imshow(imgs[4])
    plt.subplot(3,4,12)
    imshow(np.log(1 + abs(fftshift(four[4]))))
    plt.show()

Task №2.

Build a Laplacian pyramid of the image with at least five layers. The function for constructing the Laplace pyramid should use the function of constructing the Gaussian pyramid and have, like the function of the Gaussian pyramid, the parameters img, sigma and n_layers.

from skimage.io import imread, imshow
from skimage.filters import gaussian
from numpy.fft import fft2, fftshift, ifft2
import numpy as np
import matplotlib.pyplot as plt
import cv2
from skimage import img_as_float

def build_laplas_pyramid(img, sigma, n_layers):
    FOURIER_IMG = fft2(img)
    filt_gauss = np.zeros(img.shape)
    filt_gauss[img.shape[0] // 2, img.shape[1] // 2] = 1
    IMG_GAUSS = gaussian(filt_gauss, sigma=sigma)
    FOURIER_GAUSS = fft2(IMG_GAUSS)
    imgs_g = []
    fouries = []
    fouries_lap=[]
    imgs=[]
    for i in range(n_layers):
        FOURIER_BLUR_IMG = abs(FOURIER_GAUSS)* FOURIER_IMG
        FOURIER_IMG=FOURIER_BLUR_IMG
        imgs_g.append(ifft2(FOURIER_BLUR_IMG).real)
        fouries.append(FOURIER_BLUR_IMG)
    for i in range(n_layers):
        if (i-1)<0:
            fouries_lap.append(FOURIER_IMG-fouries[0])
            imgs.append(ifft2(fouries_lap[0]).real)
        else:
            fouries_lap.append(fouries[i-1]-fouries[i])
            imgs.append(ifft2(fouries_lap[i]).real)
    return imgs, fouries_lap

if __name__ =="__main__":
    src = img_as_float(imread('lena1.jpg', as_gray=True))
    plt.figure(figsize=(25, 15))
    imgs, four  = build_laplas_pyramid(src, 2, 5)
    plt.subplot(341)
    imshow(src)
    plt.subplot(342)
    FOURIER_IMG = fft2(src)
    imshow(np.log(1 + abs(fftshift(FOURIER_IMG))))
    plt.subplot(343)
    imshow(imgs[0])
    plt.subplot(344)
    imshow(np.log(1 + abs(fftshift(four[0]))))
    plt.subplot(345)
    imshow(imgs[1])
    plt.subplot(346)
    imshow(np.log(1 + abs(fftshift(four[1]))))
    plt.subplot(347)
    imshow(imgs[2])
    plt.subplot(348)
    imshow(np.log(1 + abs(fftshift(four[2]))))
    plt.subplot(349)
    imshow(imgs[3])
    plt.subplot(3,4,10)
    imshow(np.log(1 + abs(fftshift(four[3]))))
    plt.subplot(3,4,11)
    imshow(imgs[4])
    plt.subplot(3,4,12)
    imshow(np.log(1 + abs(fftshift(four[4]))))
    plt.show()

Task №3.

What levels of the Gaussian and Laplace pyramids need to be summed to get the original image? Find this levels, sum them and get the original image.

from skimage.io import imread, imshow
from skimage.filters import gaussian
from numpy.fft import fft2, fftshift, ifft2
import numpy as np
import matplotlib.pyplot as plt
import cv2
from skimage import img_as_float

def build_laplas_pyramid(img, sigma, n_layers):
    FOURIER_IMG = fft2(img)
    filt_gauss = np.zeros(img.shape)
    filt_gauss[img.shape[0] // 2, img.shape[1] // 2] = 1
    IMG_GAUSS = gaussian(filt_gauss, sigma=sigma)
    FOURIER_GAUSS = fft2(IMG_GAUSS)
    imgs_g = []
    fouries = []
    fouries_lap=[]
    imgs=[]
    for i in range(n_layers):
        FOURIER_BLUR_IMG = abs(FOURIER_GAUSS)* FOURIER_IMG
        FOURIER_IMG=FOURIER_BLUR_IMG
        imgs_g.append(ifft2(FOURIER_BLUR_IMG).real)
        fouries.append(FOURIER_BLUR_IMG)
    for i in range(n_layers):
        if (i-1)<0:
            fouries_lap.append(FOURIER_IMG-fouries[0])
            imgs.append(ifft2(fouries_lap[0]).real)
        else:
            fouries_lap.append(fouries[i-1]-fouries[i])
            imgs.append(ifft2(fouries_lap[i]).real)
    original_fories=np.zeros((src.shape[:2]))
    for i in range(5):
        original_fories=original_fories+fouries_lap[i]+fouries[0]
        img=ifft2(original_fories).real
    return img, original_fories

if __name__ =="__main__":
    src = img_as_float(imread('lena1.jpg', as_gray=True))
    plt.figure(figsize=(25, 15))
    imgs, four  = build_laplas_pyramid(src, 2, 5)
    plt.subplot(121)
    imshow(np.log(1 + abs(fftshift(four))))
    plt.subplot(122)
    imshow(imgs)
    plt.show()